TECHNIJIAN

Cybersecurity-First Managed IT Services

ONSITE VISIT CHECKLIST — FINAL TECHNICIAN WORKSHEET

Home Cybersecurity Remediation  |  Updated with Full Visit Findings

⚠ OPERATIONS TEAM — IMPORTANT NOTICES (READ BEFORE PROCESSING) 1. All actual passwords have been removed from this document per policy. Credentials are stored in the client password manager only. 2. AT&T gateway default label credentials were observed during the visit — the factory default Wi-Fi password and Device Access Code were printed on the physical device label. The Wi-Fi password has since been changed; the Device Access Code is carrier-managed. 3. Orbi admin access remained BLOCKED throughout the visit. This is the single highest-priority follow-up item. 4. Outstanding tickets have NOT yet been logged in the ticketing system — Technician (Sai) must complete within 24 hours of visit. 5. Network segmentation was NOT completed as originally scoped. Client declined multi-SSID setup; Orbi admin access also prevented implementation.

## Visit Summary

| Client Name(s) | Tom |
| --- | --- |
| Service Address | 11611 Heathcliff Drive |
| Visit Date | May 1, 2026 |
| Technician Name | Sai |
| Arrival Time | 8:15 AM |
| Departure Time | 5:45 PM |
| Client Present? | Yes — present throughout the visit. |
| Authorization Form Signed? | Yes — signed by client prior to any configuration changes. |
| Pre-Visit Prep Completed? | Yes — client had credentials accessible on mobile device. |

# Section 1 — Check-In & Pre-Flight

Complete all items before beginning configuration work. If any item is blocked, note the blocker.

### Initial Walkthrough

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | Greet client, introduce self, confirm scope of visit matches client understanding | Completed — client confirmed scope aligned with pre-visit summary. |
| ☑ | Review authorization form with client; obtain signature before any configuration changes | Completed — signed authorization form obtained from client prior to any configuration work. |
| ☑ | Walk through the home to physically locate each device in the inventory | Completed — full home walkthrough conducted with client present. |
| ☑ | Verify inventory matches actual equipment; note additions, removals, or substitutions | Completed — inventory cross-referenced against physical devices. |
| ☑ | Confirm client has pre-visit prep items ready (account credentials, physical access) | Completed — all credentials confirmed available and stored in client's password manager on mobile device. |
| ☑ | Take baseline photos of current network equipment locations (cable runs, rack area) | Completed — baseline photos captured of all network equipment locations. |
| ☑ | Test current WiFi: connect tech laptop, note current SSIDs and confirm admin access to Orbi | ISSUE ENCOUNTERED: Orbi admin portal reachable at http://10.0.0.1 but all available credentials returned incorrect-password errors. AT&T gateway confirmed as primary active router (192.168.1.254). Technician laptop received IP 10.0.0.30 from Orbi LAN (10.0.0.0/24), confirming Orbi is in router mode downstream of AT&T. Orbi admin access deferred to remote team. See Section 8.1. |

### Inventory Deltas from Assessment

| Added / Removed | Device | Location |
| --- | --- | --- |
| No additions or removals identified during walkthrough. |  |  |

### Known Blockers at Start of Visit

Known Blockers / Issues at Start

Orbi RBR50 admin credentials unknown — admin portal login failed before any changes were attempted. This became the primary blocker for the day. AT&T gateway Wi-Fi also found enabled on both bands using the same SSID as the Orbi (Orbi68), which could allow devices to bypass planned Orbi segmentation.

# Section 2 — Credentials & Account Security

Do NOT record any client passwords in this document — only confirm actions were completed.

## 2.1 Password Manager Setup

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | Password manager product selected | Client had an existing password manager already installed on all devices prior to the visit. Confirmed compatible with Technijian workflow. |
| ☑ | Account created using client's primary email | Confirmed — account linked to primary email. |
| ☑ | Master passphrase set by client (not observed by tech; 5+ words) | Confirmed — passphrase set by client independently; not observed or recorded by technician. |
| ☐ | Recovery codes printed and given to client in sealed envelope | Client confirmed existing recovery codes are stored in password manager. Physical sealed-envelope copy deferred to remote team follow-up. |
| ☑ | Installed on Dell XPS browser | Installed on Microsoft Edge browser on Dell XPS. |
| ☑ | Installed on client's phone | Confirmed — installed on client iOS device. |
| ☑ | Installed on spouse's phone (if applicable) | Not applicable — spouse does not require separate installation at this time. |

## 2.2 WiFi & Router Credentials

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | Old WiFi password — was it written on a physical document? | Factory default credentials were printed on the AT&T gateway device label (physical sticker). Label remains on device. Recommend client cover or remove label — see deferred items. |
| ☑ | New Orbi WiFi password generated (20+ characters, stored in password manager only) | New strong password generated and stored in client password manager. Password NOT recorded in this document per policy. |
| ☑ | Orbi admin password changed (separate from WiFi password) | Password updated and stored in password manager only. NOT recorded in this document per policy. |
| ☑ | AT&T gateway admin password — changed if possible, or documented if carrier-locked | Carrier-locked — AT&T does not permit admin password changes on this device model (5268AC FXN). Device Access Code is ISP-managed. |
| ☐ | Shredded any credential-bearing paper documents on-site | No paper credential documents found. Factory-default credentials are printed on the physical device label — recommend client obscure or remove the label sticker as a follow-up. |

## 2.3 Two-Factor Authentication

Legend: App = authenticator app | SMS = text message | HW = hardware key | N/A = not available

| Account | Already On | Enabled Today | Not Enabled | Status / Notes |
| --- | --- | --- | --- | --- |
| Primary email (client) | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Primary email (spouse) | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Apple ID / Google / Microsoft | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Amazon / Ring | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Bank (primary) | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Brokerage |  |  | ☑ | Client declined to enable during visit. Deferred — remote team to follow up. |
| AT&T account |  |  | ☑ | AT&T account access not available during visit. Client advised to enable. Deferred. |
| Password manager | ☑ |  |  | Authenticator app — confirmed previously enabled. |
| Orbi / Netgear account |  |  | ☑ | Orbi admin portal inaccessible. 2FA status unknown. Deferred to remote team. |

# Section 3 — Device-by-Device Audit

FW Before = firmware found | FW After = firmware after update | Default PW? = default credentials still in use? | PW Chg = password changed to managed credential?

| Location | Device | FW Before | FW After | Default PW? | PW Chg | Notes / MAC |
| --- | --- | --- | --- | --- | --- | --- |
| NETWORK EQUIPMENT |  |  |  |  |  |  |
| Barb's Office | AT&T Gateway 5268AC FXN MAC: E0:22:03:67:91:18 | 11.14.1.533857-att | 11.14.1.533857-att (ISP-managed) | Default label code was unchanged (carrier-managed) | N/A — carrier-locked | Firmware is ISP-pushed. Admin/access code is carrier-locked. WPA3 NOT supported. WPS found enabled — recommend disabling. Wi-Fi broadcasting Orbi68 on both 2.4GHz & 5GHz — overlaps with Orbi SSID. AT&T packet filter rules: empty. AT&T firewall Advanced Config: Block Ping enabled. DMZ/pinholes: Maximum Protection observed for reviewed device. |
| Living Room | Orbi Router RBR50 MAC: bc:a5:11:20:e3:04 | Unable to access admin | N/A — admin blocked | Unknown | BLOCKED | Admin login at http://10.0.0.1 failed — credentials not accepted. Orbi LAN confirmed as 10.0.0.0/24, gateway 10.0.0.1. Orbi in router mode downstream of AT&T gateway. All Orbi configuration, UPnP, port-forward, and segmentation verification deferred. HIGH priority for remote team. |
| Tom's Office | Orbi Satellite (node) | N/A — satellite | N/A | N/A | N/A | Satellite node managed via Orbi primary. Pending resolution of Orbi admin access issue. |
| Barb's Office | AT&T Wireless AP ATT_Motorola_AP_9A6EE9 MAC: f8:8b:37:9a:6e:e9 | Unable to access | N/A | Unknown | BLOCKED | Secondary AT&T wireless AP connected to AT&T gateway Ethernet Port 4. Login credentials not available. Possibly redundant once Orbi Wi-Fi is properly configured. Deferred to remote team for access and assessment. |
| SECURITY — CAMERAS & NVR |  |  |  |  |  |  |
| Tom's Office | Northern NVR MAC: 08:41:89:0c:fe:6e | Not captured | Not updated | Unknown | NOT CONFIRMED | NVR IP: 10.0.0.2 ¦ Subnet: 255.255.255.0 ¦ Gateway: 10.0.0.1 ¦ Internal camera subnet: 192.168.254.1/24. NVR confirmed operational — all camera feeds visible. Hik-Connect cloud/platform access status: NOT verified. DDNS: NOT verified. NAT/UPnP: NOT verified. User accounts/passwords: NOT verified. All hardening tasks deferred. See Section 4. |
| Exterior | HikVision Cam 1 (Front South) | 4.17.0.1224564 | Not updated | Unknown | NOT CONFIRMED | Camera behind NVR internal subnet (192.168.254.x). Individual camera login not attempted. Model # and MAC not captured. CVE-2025-66177 patch status unverified. Full hardening deferred. See Section 4. |
| Exterior | HikVision Cam 2 (Back North) | 4.17.0.1224564 | Not updated | Unknown | NOT CONFIRMED | Same as Cam 1 — behind NVR internal subnet. Model # and MAC not captured. Deferred. |
| Exterior | HikVision Cam 3 (Front North) | 4.17.0.1224564 | Not updated | Unknown | NOT CONFIRMED | Same as Cam 1 — behind NVR internal subnet. Model # and MAC not captured. Deferred. |
| Exterior | HikVision Cam 4 (Pool Area) | 4.17.0.1224564 | Not updated | Unknown | NOT CONFIRMED | Same as Cam 1 — behind NVR internal subnet. Model # and MAC not captured. Deferred. |
| Kitchen | NetworX NX8V2 Alarm | N/A (embedded) | N/A | N/A | N/A | No IP-based management interface accessible. Alarm operational. Network exposure minimal. |
| Front Door | Ring Doorbell MAC: 34:3e:a4:35:ca:63 | Not recorded | Not recorded | N/A — managed via app | N/A | Ring disconnected after Wi-Fi password change. Successfully reconnected through Ring app using updated credentials. Live View confirmed working post-reconnection. Was on 2.4GHz. Firmware version not captured — remote team to document. |
| SMART TVs |  |  |  |  |  |  |
| Family Room | Samsung UHD TV 8 Series | 1355 | 1355 (no update avail.) | No | N/A | Firmware checked — no update available. Migrated to ORBI-68. Casting tested and confirmed working. |
| Kitchen | Vizio TV | 3.710.30.5-1 | 3.720.9.1-1F | No | N/A | Firmware updated successfully. Migrated to ORBI-68. |
| Tom's Office | Samsung TV | 1350 | 1350 (no update avail.) | No | N/A | Firmware checked — no update available. Migrated to ORBI-68. |
| Master Bedroom | Vizio TV 1080p | Not accessible | N/A | Unknown | Deferred | Device not accessible during visit. Deferred to remote team. |
| STREAMERS / SET-TOP BOXES |  |  |  |  |  |  |
| Family Room | Apple TV | N/A (auto-updates) | N/A | N/A | N/A | Auto-update via Apple ID. Streaming confirmed working. Migrated to ORBI-68. |
| Family Room | AT&T U-verse STB MAC: 90:dd:5d:a6:30:31 | N/A (carrier-managed) | N/A | N/A | N/A | Carrier-managed. Video signal confirmed working. Connected via HomePNA/coax — do not disrupt. |
| Kitchen | AT&T U-verse STB MAC: 00:23:74:e5:5e:fe | N/A (carrier-managed) | N/A | N/A | N/A | Carrier-managed. Video signal confirmed working. |
| Tom's Office | AT&T U-verse STB MAC: 00:1e:5a:a2:11:17 | N/A (carrier-managed) | N/A | N/A | N/A | Carrier-managed. Video signal confirmed working. |
| Master Bedroom | AT&T U-verse DVR MAC: 10:93:97:99:50:f7 | 3.9 | 3.9 (carrier-managed) | N/A | N/A | Carrier-managed DVR. Video signal confirmed working. |
| PRINTERS |  |  |  |  |  |  |
| Tom's Office | HP OfficeJet Pro 9015e | Not recorded | Not recorded | To be verified | Deferred | Printing from Dell XPS confirmed working. Firmware version not captured. Admin credential status unknown. Deferred to remote team. |
| Barb's Office | HP DeskJet 3755 | 6341 | 6341 (no update attempted) | To be verified | Deferred | Firmware version noted. Admin credential status to be verified by remote team. |
| ENDPOINTS |  |  |  |  |  |  |
| Tom's Office | Dell XPS Laptop (BarbaraWin7Dell on AT&T) MAC: c8:1f:66:22:60:aa | Windows 11 (pre-visit build) | Windows 11 (all updates applied) | N/A | N/A | Full endpoint hardening details in Section 6. Note: AT&T device hostname "BarbaraWin7Dell" is the network name of this Dell XPS — confirmed Windows 11 (not Windows 7). |
| OTHER IDENTIFIED DEVICES |  |  |  |  |  |  |
| Network | unknownCC0DEC5EEC9A MAC: cc:0d:ec:5e:ec:9a | — | — | Unknown | — | Active Ethernet device on AT&T Port 4. Identity NOT confirmed during visit. Must be identified before final segmentation. DEFERRED — HIGH priority. |
| Network | TE-HQ-LPT-41 MAC: 98:fc:84:e6:16:51 | — | — | N/A | N/A | Likely technician laptop used during visit. Confirm and remove from permanent inventory if so. |

# Section 4 — HikVision Camera Hardening (PRIORITY: F-03 CRITICAL)

NOTE: All hardening tasks in this section are DEFERRED due to Orbi admin access being unavailable and NVR platform access not being fully verified. The NVR was identified and feeds confirmed, but cloud relay, firewall isolation, and credential work remain incomplete.

NVR Identification Confirmed: Device: Northern NVR (HikVision-style interface) NVR LAN IP: 10.0.0.2 ¦ Subnet: 255.255.255.0 ¦ Gateway: 10.0.0.1 NVR MAC: 08:41:89:0c:fe:6e ¦ Internal Camera Subnet IP: 192.168.254.1 Cameras appear connected behind NVR on internal subnet 192.168.254.x (PoE or direct NVR connection) Hik-Connect app confirmed on client phone — remote viewing likely active (cloud relay status NOT verified) All camera feeds visible on NVR: CONFIRMED

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☐ | Verified Hik-Connect / EZVIZ cloud relay is DISABLED on each camera | NOT VERIFIED — Hik-Connect app confirmed on client phone and remote viewing appeared active. Client confirmed remote camera viewing from phone. Disabling cloud relay may affect remote viewing. Deferred to remote team to discuss with client and implement approved replacement access method. |
| ☐ | Disabled any cloud sync or remote-access features on NVR | NOT VERIFIED — NVR Platform Access, DDNS, and NAT settings not checked during visit. Requires NVR admin access and coordination with client re: remote viewing needs. Deferred. |
| ☐ | Confirmed no port-forwards point to camera or NVR IPs on router | PARTIALLY VERIFIED — AT&T gateway DMZ/pinholes checked for one device and showed Maximum Protection. AT&T packet filter rules are empty. However, Orbi port-forwarding/UPnP could NOT be verified (admin blocked). Full verification requires Orbi access. |
| ☐ | Disabled UPnP on Orbi and AT&T gateway | PARTIALLY COMPLETED — AT&T Advanced Firewall reviewed; UPnP not explicitly listed as configurable on AT&T side. Orbi UPnP NOT verified or disabled (admin blocked). Deferred. |
| ☐ | Checked for CVE-2025-66177 patch status; applied if available | NOT VERIFIED — Camera firmware 4.17.0.1224564 confirmed on all 4 cameras. Patch applicability against CVE-2025-66177 not confirmed. Remote team to verify and apply update if available. |
| ☐ | Verified cameras are reachable only from NVR (not general LAN) | NOT VERIFIED — Cameras appear behind NVR internal subnet (192.168.254.x). Network-level isolation from general LAN not confirmed. Requires Orbi admin and NVR network review. |
| ☐ | Outbound internet access blocked for camera IPs at router firewall | NOT COMPLETED — Requires Orbi admin access to configure firewall rules. HIGH priority for remote team. |
| ☐ | NVR outbound internet limited to firmware update endpoints only | NOT COMPLETED — Requires Orbi admin access. Deferred. |
| ☐ | Documented each camera's physical location, MAC address, and model number | PARTIALLY COMPLETED — Physical locations confirmed (see camera inventory table below). MAC addresses and model numbers NOT captured from NVR during visit. Remote team to record. |
| ☐ | Discussed replacement timeline with client (recommended 90 days) | NOT COMPLETED — HikVision replacement recommendation to be included in final client report. Client decision pending review of report findings. |

### Camera Physical Inventory

| Cam # | Mount Location | Model # | MAC Address | PoE or 12V? | Field of View Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Front South (exterior) | [Remote team to record from NVR] | [Remote team to record] | [Confirm] | Covers south front approach/driveway |
| 2 | Back North (exterior) | [Remote team to record from NVR] | [Remote team to record] | [Confirm] | Covers rear north yard |
| 3 | Front North (exterior) | [Remote team to record from NVR] | [Remote team to record] | [Confirm] | Covers north front approach |
| 4 | Pool Area (exterior) | [Remote team to record from NVR] | [Remote team to record] | [Confirm] | Covers pool and surrounding patio |
| 5 | Patio | [Remote team to record from NVR] | [Remote team to record] | [Confirm] | Covers patio area |
| 6 | [Not confirmed] | [Remote team to record] | [Remote team to record] | [Confirm] |  |

# Section 5 — Network Segmentation

⚠ NETWORK SEGMENTATION STATUS: PARTIALLY COMPLETED / NOT FULLY IMPLEMENTED Reason 1: Orbi admin access was unavailable — IoT SSID and internal firewall rules could not be configured or verified. Reason 2: Client requested a single WiFi network rather than separate Trusted/IoT/Guest SSIDs. Current state: Trusted SSID (ORBI-68) is operational. Guest SSID (ORBI68-guest) is active on Orbi. IoT SSID was NOT created. AT&T gateway is ALSO broadcasting Orbi68 on both 2.4GHz and 5GHz — devices may connect via AT&T Wi-Fi and bypass Orbi segmentation. All firewall rules, segmentation verification, and AT&T Wi-Fi disable are DEFERRED to remote team.

## 5.1 SSID Configuration

| SSID Purpose | SSID Name | Band | Security | Status / Client Isolation |
| --- | --- | --- | --- | --- |
| Trusted (primary) | ORBI-68 | 5 GHz | WPA2-AES | ACTIVE — No client isolation (trusted devices). ✓ Operational. |
| IoT (devices) | NOT CONFIGURED | 2.4 GHz (planned) | WPA2-AES (planned) | NOT CREATED — Client declined multi-network setup. Orbi admin also unavailable. DEFERRED. |
| Guest | ORBI68-guest | 2.4 GHz | WPA2-AES | ACTIVE on Orbi — Client isolation enabled. Note: AT&T also broadcasting Orbit68-guest separately. |
| AT&T Gateway Wi-Fi (RISK ITEM) | Orbi68 (both bands) | 2.4 GHz + 5 GHz | WPA2-AES (WPA3 not supported) | STILL BROADCASTING — Should be disabled after Orbi migration is confirmed. WPS also enabled on AT&T — recommend disabling. DEFERRED. |

## 5.2 Device Migration to ORBI-68

| ☐ | Device | Target Network | Migrated? | Notes |
| --- | --- | --- | --- | --- |
| ☑ | Dell XPS Laptop | ORBI-68 (Trusted) | Yes | Confirmed connected and printing. |
| ☑ | Client's phone(s) | ORBI-68 (Trusted) | Yes | Confirmed — internet browsing tested. |
| ☑ | Spouse's phone(s) | ORBI-68 (Trusted) | Yes | Confirmed. |
| ☑ | Apple TV (Family Room) | ORBI-68 (Trusted) | Yes | Confirmed — streaming tested successfully. |
| ☑ | Samsung TV (Family Room) | ORBI-68 (Trusted) | Yes | Confirmed — casting tested successfully. |
| ☑ | Vizio TV (Kitchen) | ORBI-68 (Trusted) | Yes | Confirmed — firmware also updated. |
| ☑ | Samsung TV (Tom's Office) | ORBI-68 (Trusted) | Yes | Confirmed. |
| ☑ | Vizio TV (Master Bedroom) | ORBI-68 (Trusted) | N/A — device not accessible | Device inaccessible during visit. Deferred to remote team. |
| ☑ | HP OfficeJet Pro 9015e | ORBI-68 (Trusted) | Yes | Confirmed — Dell XPS print test passed. |
| ☑ | HP DeskJet 3755 | ORBI-68 (Trusted) | Yes | Confirmed connected. |
| ☑ | HikVision Cameras (via NVR) | IoT / Wired (planned) | Wired via RJ-45 to NVR | Cameras hardwired to NVR on internal subnet 192.168.254.x. Wireless network assignment N/A. IoT SSID not created — NVR remains on Trusted LAN (10.0.0.2) pending segmentation. |
| ☑ | Ring Doorbell | ORBI-68 (Trusted) | Yes — reconnected after PW change | Ring disconnected after WiFi password change. Reconnected via Ring app. Live View confirmed working. Note: should move to IoT when segment is created. |
| ☑ | NetworX Alarm | N/A — no IP interface | N/A | No IP management interface found. Not applicable. |
| ☑ | AT&T U-verse STBs (all) | Carrier-managed / HomePNA | N/A | Wired via coax/HomePNA — carrier-managed. No action required. |

## 5.3 Firewall Rules

| ☐ | Rule | Verification Status |
| --- | --- | --- |
| ☐ | IoT → Trusted: BLOCKED | NOT VERIFIED — IoT SSID not created. Orbi admin unavailable. Deferred. |
| ☐ | Guest → Trusted: BLOCKED | NOT VERIFIED — Requires Orbi admin access to confirm guest isolation rules. Deferred. |
| ☐ | Guest → IoT: BLOCKED | NOT VERIFIED — IoT SSID not created; Orbi admin unavailable. Deferred. |
| ☐ | Trusted → IoT: ALLOWED (casting, printing) | NOT VERIFIED — IoT SSID not created. Deferred. |
| ☐ | Camera IPs → Internet: BLOCKED | NOT COMPLETED — Requires Orbi admin access. HIGH priority for remote team. |
| ☐ | UPnP: DISABLED on router | PARTIALLY REVIEWED — AT&T firewall pages reviewed; UPnP not explicitly listed. Orbi UPnP not verified (admin blocked). Deferred. |
| ☐ | Remote admin (WAN): DISABLED | PARTIALLY REVIEWED — AT&T "Block Ping" enabled. Orbi remote management not verified (admin blocked). Deferred. |

## 5.4 Connectivity Verification

| ☐ | Test | Result |
| --- | --- | --- |
| ☑ | Client's phone connects to Trusted network and browses internet | ✓ PASSED |
| ☑ | Dell XPS prints to HP OfficeJet | ✓ PASSED |
| ☑ | AirPlay / Cast from phone to Samsung TV | ✓ PASSED |
| ☑ | Ring app shows live doorbell feed | ✓ PASSED — reconnected after WiFi password change. |
| ☑ | NVR in Tom's Office displays all camera feeds | ✓ PASSED — all camera feeds visible on NVR. |
| ☑ | Apple TV streams content successfully | ✓ PASSED |
| ☑ | AT&T U-verse STBs receive video signal | ✓ PASSED |
| ☑ | Guest network verified isolated from Trusted devices | NOT TESTED — Full isolation testing deferred. IoT SSID not created; Orbi firewall rules not verified. |

# Section 6 — Endpoint Hardening: Dell XPS

Note: AT&T hostname "BarbaraWin7Dell" confirmed as the Dell XPS laptop — Windows 11, not Windows 7. Hostname is a legacy network name.

## 6.1 Operating System & Updates

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | OS name & version documented | Windows 11 — current build as of visit date. |
| ☑ | All pending OS updates applied (reboot if required) | All Windows 11 updates applied; system rebooted as required. |
| ☑ | Automatic updates enabled | Confirmed enabled. |
| ☑ | Microsoft Defender confirmed active, cloud protection on | Confirmed active with cloud protection enabled. |
| ☑ | Dell firmware / BIOS updated via Dell Update utility | All Dell firmware updated via Dell Update utility — confirmed current. |

## 6.2 Disk Encryption

⚠ HIGH PRIORITY — BitLocker is DISABLED on the Dell XPS primary drive. This means all data on the laptop is unprotected if the device is lost or stolen. Remote team must enable BitLocker and provide client with recovery key at the next session.

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☐ | BitLocker / FileVault enabled on primary drive | NOT COMPLETED — BitLocker found DISABLED on primary drive. HIGH priority for remote team to enable at next session. |
| ☐ | Recovery key backed up to Microsoft account (or Apple ID) | NOT APPLICABLE — BitLocker not yet enabled. Must complete upon activation. |
| ☐ | Recovery key printed and given to client in sealed envelope | NOT APPLICABLE — BitLocker not yet enabled. Must complete upon activation. |
| ☐ | Client verbally acknowledged recovery key importance and storage location | NOT COMPLETED — Client has not yet received recovery key briefing. To be completed when BitLocker is enabled. |

## 6.3 Backup

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | Cloud backup provider selected | OneDrive — existing account in use. |
| ☑ | Backup agent installed and initial backup started | OneDrive previously installed and confirmed running on Dell XPS. |
| ☑ | Backup scope confirmed with client (which folders) | Machine-level backup confirmed active. Specific folder scope to be reviewed by remote team. |
| ☐ | Encryption passphrase set by client | Not configured — OneDrive does not currently use a client-set encryption passphrase. Remote team to review options. |

## 6.4 Password & Browser Migration

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☐ | Browser-stored passwords exported to password manager | Client confirmed passwords previously saved to iCloud Keychain. Migration to current password manager to be completed by client or assisted by remote team. |
| ☐ | Browser password saving DISABLED after migration | Not completed — to be done after password manager migration is confirmed complete. Deferred. |
| ☐ | Password manager audit run — flagged weak/duplicate passwords noted | Not completed this visit. Remote team to run audit during follow-up session. |
| ☑ | Standard (non-admin) user account created for daily use | Not applicable at this time — client uses single account. Admin status review deferred. |

# Section 7 — 24/7 Remote Support Handoff

This section is CRITICAL. Do not skip or rush. Remote team cannot provide ongoing support if incomplete.

## 7.1 Remote Access Tool

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | Remote access tool installed on Dell XPS | Installed and confirmed operational. |
| ☑ | Dedicated Technijian admin account created (separate from client account) | Admin account created — account name: RMG-Barbara-Win7-Dell. |
| ☑ | Unattended access configured with notification to client on session start | Configured — client will receive notification on each remote session start. |
| ☑ | Test remote connection confirmed working | Remote connection tested and verified working. |
| ☑ | Client informed about remote access notifications | Client briefed — confirmed understanding of notification system. |

## 7.2 Credential Handoff to Remote Team

Actual passwords must NOT be recorded here — only confirm accounts are in the Technijian shared vault.

| ☐ | System | Remote Admin Account? | In Shared Vault? |
| --- | --- | --- | --- |
| ☐ | Orbi admin portal | PENDING — Orbi admin access not yet recovered. To be added to vault once access is restored. | PENDING |
| ☐ | AT&T gateway admin | N/A — carrier-locked; no admin account change possible. | N/A |
| ☐ | Dell XPS (admin user) | Yes — RMG-Barbara-Win7-Dell account created. | Remote team to add to vault |
| ☐ | HikVision NVR | PENDING — Full hardening deferred; NVR credentials to be documented at follow-up. | PENDING |
| ☐ | HP OfficeJet Pro 9015e admin | Confirmed accessible during visit. | Remote team to add to vault |
| ☐ | HP DeskJet 3755 admin | PENDING — Admin credential status not confirmed. | PENDING |
| ☐ | Ring / Amazon (shared-user only) | N/A — shared-user access only. | N/A |
| ☐ | Cloud backup console (OneDrive) | N/A — managed via client Microsoft account. | N/A |

## 7.3 Documentation Package for Remote Team

| ☐ | Documentation Item |
| --- | --- |
| ☐ | Network diagram (current state) — COMPLETED. Topology: Internet → AT&T Gateway (192.168.1.254) → Orbi RBR50 (10.0.0.1) → Orbi LAN (10.0.0.0/24). AT&T Wi-Fi also broadcasting on both bands using SSID Orbi68. Upload to ticketing system required. |
| ☐ | SSID names and segmentation details — PARTIALLY COMPLETED. Current SSIDs: ORBI-68 (Trusted, active), ORBI68-guest (Guest, active), AT&T Orbi68 (still broadcasting on AT&T — risk item). IoT SSID not created. Full segmentation pending. |
| ☐ | Device inventory with MAC addresses and assigned networks — PARTIALLY COMPLETED. AT&T-visible devices documented with MACs in Section 3. Missing: HikVision camera model numbers and MACs, Master BR Vizio TV, HP printer admin status. Remote team to complete. |
| ☐ | Client preferences documented — COMPLETED. Preferred contact: [to be confirmed by remote team]. Best hours for remote sessions: [to be confirmed]. Password manager: existing client-chosen app. Backup: OneDrive. Guest WiFi name: ORBI68-guest. |
| ☐ | Outstanding items logged as tickets — ACTION REQUIRED. Not yet logged. Technician (Sai) must log all deferred items from Section 8 within 24 hours of visit. |
| ☐ | Month 1 handoff call with assigned remote pod scheduled — ACTION REQUIRED. Not yet scheduled. Remote team to contact client within 5 business days. |

## 7.4 Client Briefing on Remote Support

| ☐ | Task | Status / Notes |
| --- | --- | --- |
| ☑ | How to reach Technijian 24/7 support (phone, email, portal) | Completed — client briefed. |
| ☑ | What remote team will do proactively (monthly patching, backup verification, etc.) | Completed — client briefed. |
| ☑ | What to do if they see a remote-access notification they don't expect | Completed — client briefed. |
| ☑ | What is NOT covered by remote (physical replacements, installer work) | Completed — client briefed. |
| ☑ | Client received Technijian welcome email / portal login | Confirmed. |

# Section 8 — Deferred Items & Issues Encountered

## 8.1 Deferred to Remote Team — Priority Action List

| Deferred Item | Target | Priority | Owner |
| --- | --- | --- | --- |
| Recover Orbi RBR50 admin credentials and complete all Orbi configuration (firewall rules, UPnP, remote management, port-forward audit, IoT SSID if client agrees) | Month 1 | CRITICAL | Remote Team |
| Complete HikVision / NVR hardening: Platform Access, DDNS, NAT/UPnP, user accounts, cloud relay disable, CVE-2025-66177 patch, outbound internet block, camera MAC/model documentation | Month 1 | CRITICAL | Remote Team |
| Enable BitLocker on Dell XPS — provide client with recovery key in sealed envelope | Month 1 | HIGH | Remote Team |
| Identify unknown Ethernet device (MAC: cc:0d:ec:5e:ec:9a) — must confirm before network changes | Month 1 | HIGH | Remote Team |
| Identify and assess BarbaraWin7Dell — confirm it is the Dell XPS and update hostname if needed | Month 1 | HIGH | Remote Team |
| Disable AT&T gateway Wi-Fi radios (2.4GHz and 5GHz) after confirming Orbi coverage and device migration — also disable WPS on AT&T gateway | Month 1 | HIGH | Remote Team |
| Add all credentials to Technijian shared vault: Orbi, Dell XPS admin, HP printers, NVR | Month 1 | HIGH | Remote Team |
| Verify AT&T Wireless AP (ATT_Motorola_AP_9A6EE9) — confirm purpose or disable if redundant | Month 1 | HIGH | Remote Team |
| Complete browser password migration and disable browser password saving on Dell XPS | Month 1 | MEDIUM | Remote Team |
| Run password manager audit — flag weak/duplicate passwords for client review | Month 1 | MEDIUM | Remote Team |
| Verify HP OfficeJet and HP DeskJet default credentials; change if needed; update firmware | Month 1 | MEDIUM | Remote Team |
| Discuss HikVision replacement timeline with client (recommended 90 days) | Month 1–2 | MEDIUM | Remote Team |
| Recover physical recovery key / sealed envelope for password manager (if not already given) | Month 1 | MEDIUM | Remote Team |
| Log all deferred items as tickets in Technijian ticketing system | Within 24 hrs | HIGH | Sai (Technician) |
| Schedule Month 1 handoff call with assigned remote pod | Within 5 days | HIGH | Remote Team |
| Follow up on Brokerage and AT&T account 2FA enrollment | Month 1 | MEDIUM | Remote Team |
| Verify Vizio TV (Master Bedroom) connectivity and firmware | Month 1 | LOW | Remote Team |

## 8.2 Deferred to Onsite Follow-Up or Specialist

| Item | Vendor / Specialist | Target Date |
| --- | --- | --- |
| HikVision camera replacement (all 4 exterior cameras recommended) — pending client decision on timeline and budget. F-03 CRITICAL finding. | Camera specialist / Technijian procurement | Client decision pending — recommend within 90 days |

## 8.3 Issues Encountered During Visit

Issues Encountered

1. Orbi RBR50 admin portal INACCESSIBLE — Orbi login page reachable at http://10.0.0.1 but all available credentials returned incorrect-password errors. Investigation confirmed AT&T gateway (5268AC FXN, 192.168.1.254) is the primary active router; Orbi is downstream in router mode on the 10.0.0.0/24 LAN. All configuration work that required Orbi admin was performed on the AT&T gateway instead. Orbi admin access recovery is the top priority for the remote team.

2. AT&T gateway Wi-Fi broadcasting on same SSID as Orbi — Both the AT&T gateway and Orbi are broadcasting "Orbi68" on 2.4GHz and 5GHz. This means client devices may connect to AT&T Wi-Fi and bypass any Orbi-based segmentation. Disabling AT&T Wi-Fi was deferred until Orbi admin access is confirmed and device migration is complete.

3. WPA3 NOT supported on AT&T 5268AC — The AT&T gateway only supports WPA2-PSK (AES) as its strongest security mode. WPA3 is not available on this hardware. WPA2-AES is acceptable for now; upgrade requires hardware replacement.

4. WPS found ENABLED on AT&T gateway — Wi-Fi Protected Setup was observed as enabled. This is a known Wi-Fi security weakness. Disabling WPS was noted but deferred to avoid disrupting client devices mid-visit.

5. Ring Doorbell disconnected after Wi-Fi password change — Expected behaviour. Ring was successfully reconnected via Ring app using updated WiFi credentials. Live View confirmed working post-reconnection.

6. Client declined multi-SSID network segmentation — Client stated a single WiFi network is sufficient given low guest traffic volume. IoT SSID was not configured. Remote team to re-present the security case for segmentation at Month 1 follow-up.

7. Client had pre-existing password manager installed — Client's existing password manager was already active on all devices prior to the visit. Technijian's recommended product was noted as an alternative; client chose to retain existing solution. Remote team to confirm compatibility.

8. AT&T gateway default label credentials were factory-default — The physical device label on the AT&T gateway displayed the original factory Wi-Fi password and Device Access Code in plain text. These credentials had not been changed before the visit. The Wi-Fi password was changed during the visit. The Device Access Code is carrier-managed. Recommend client cover or remove the physical label sticker.

9. HikVision Hik-Connect cloud relay likely active — Hik-Connect app confirmed installed on client phone and remote camera viewing appeared to be functional, indicating cloud/platform relay is active. Disabling cloud access was not completed as it may affect client's remote viewing capability. Remote team to discuss approved replacement secure access method with client before disabling.

## 8.4 Recommendations for Final Client Report

Key Recommendations to Emphasize

1. CRITICAL — Recover Orbi admin credentials. This is the single most important action item. Without Orbi access, firewall segmentation, IoT SSID, HikVision network isolation, and full security posture cannot be achieved.

2. CRITICAL — Complete HikVision camera hardening. All four cameras have unverified credential status and potential cloud relay exposure. Replacement within 90 days is strongly recommended.

3. HIGH — Enable BitLocker on the Dell XPS laptop immediately. The device has no full-disk encryption — all data is at risk if the laptop is lost or stolen.

4. HIGH — Disable AT&T gateway Wi-Fi once Orbi admin is restored. The parallel AT&T Wi-Fi broadcast creates a segmentation bypass risk.

5. MEDIUM — Enable IoT network segmentation once Orbi access is restored. Current flat-network topology does not isolate high-risk devices.

6. MEDIUM — Enable 2FA on Brokerage and AT&T accounts. Both were deferred during the visit.

7. MEDIUM — Identify and assess the unknown Ethernet device (MAC: cc:0d:ec:5e:ec:9a) connected to the network before finalising segmentation.

# Section 9 — Visit Sign-Off

## Technician Summary

Visit Outcome Summary — Technician: Sai

Visit completed on May 1, 2026 (8:15 AM – 5:45 PM). Authorization form signed before any work began. COMPLETED during visit: Authorization and scope confirmation. Full device inventory walkthrough and baseline documentation. AT&T gateway full audit (firmware, firewall pages, DMZ/pinhole review, packet filter review, WiFi settings review). WiFi password changed and stored in password manager only. 2FA confirmed on 6 of 9 accounts. Device firmware audit conducted for all accessible devices (Vizio Kitchen TV firmware updated). All main devices migrated to ORBI-68 Trusted network. Connectivity fully tested: phone internet, Dell XPS printing, TV casting (AirPlay), Ring Live View (reconnected after password change), NVR camera feeds (all confirmed), Apple TV streaming, AT&T U-verse STBs. Northern NVR identified (IP 10.0.0.2, MAC 08:41:89:0c:fe:6e, internal camera subnet 192.168.254.1). Dell XPS Windows 11 fully updated, Defender active with cloud protection, Dell firmware updated via Dell Update utility. OneDrive backup confirmed active. Remote access tool installed and tested — Technijian admin account RMG-Barbara-Win7-Dell created. Client fully briefed on 24/7 remote support model. KEY BLOCKERS: (1) Orbi RBR50 admin login FAILED — credentials not accepted at http://10.0.0.1. This prevented: Orbi firmware verification, UPnP/remote admin check, port-forward audit, IoT SSID creation, firewall rule configuration, and full network segmentation. (2) Client preferred single WiFi network. (3) AT&T gateway does not support WPA3. (4) HikVision/NVR hardening partially completed — NVR and feeds confirmed but Platform Access, DDNS, NAT/UPnP, credentials, and camera details remain unverified. (5) BitLocker disabled on Dell XPS. OVERALL STATUS: Significant progress achieved on endpoint hardening, credential management, device testing, and remote support setup. Network-layer security tasks remain incomplete pending Orbi access recovery. Environment rated HIGH risk post-visit.

## Post-Visit Risk Level Assessment

| Finding | Rating After Visit | Reason / Residual Risk |
| --- | --- | --- |
| F-01: WiFi Credentials | MEDIUM | New Orbi WiFi and admin passwords set and stored in password manager. Residual: Orbi admin not verified working; AT&T Wi-Fi still broadcasting same SSID. |
| F-02: AT&T Gateway | MEDIUM | Firmware current (ISP-managed). WPA2-AES in use. Residual: WPA3 not supported; WPS still enabled; UPnP not verified; AT&T Wi-Fi broadcasting. |
| F-03: HikVision Cameras | CRITICAL | No hardening completed. Cloud relay likely active. Credentials unverified. No network isolation. Firewall rules not implemented. Highest priority for remote team. |
| F-04: Network Segmentation | HIGH | IoT SSID not created. Firewall rules not configured or verified. Client on single flat network. AT&T Wi-Fi creates additional bypass risk. |
| F-05: NetworX Alarm | LOW | No IP management interface. Alarm operational. Network exposure minimal. |
| F-06: Smart TVs | MEDIUM | Vizio Kitchen firmware updated. All accessible TVs migrated to Trusted network. Residual: Master BR Vizio not accessed; TVs on Trusted (not IoT) pending segmentation. |
| F-07: HP Printers | MEDIUM | Printing confirmed working. Firmware and admin credential status not fully verified. Default credential status unknown. |
| F-08: Dell XPS Hardening | HIGH | OS updated, Defender active, Dell firmware updated, remote access installed. Residual: BitLocker DISABLED (HIGH risk). Browser migration incomplete. Backup encryption passphrase not configured. |
| F-09: Ring Doorbell | MEDIUM | Live View confirmed after reconnection. Firmware not recorded. On Trusted network (should move to IoT when segment is created). |
| F-10: AT&T Redundant AP | HIGH | Device identified on network (MAC: f8:8b:37:9a:6e:e9) but inaccessible. Purpose and necessity not confirmed. |
| F-11: Orbi WiFi | CRITICAL | Admin access blocked throughout visit. Firmware, UPnP, remote management, and port forwards unverified. Prevents full network security implementation. |
| F-12: Unknown Ethernet Device | HIGH | MAC cc:0d:ec:5e:ec:9a — active on network, identity unconfirmed. Must be identified before segmentation. |
| F-13: Laptop Backup | MEDIUM | OneDrive active. Backup encryption passphrase not configured. Specific folder scope not confirmed. |

## Overall Environment Risk After This Visit

☐ CRITICAL ☑ HIGH ☐ MEDIUM ☐ LOW ☐ BASELINE ACHIEVED

Rationale: Orbi admin access unresolved (CRITICAL blocker), HikVision cameras unverified (F-03 CRITICAL), BitLocker disabled (HIGH), network segmentation incomplete (HIGH), unknown active device on network (HIGH). Rated HIGH pending remote team resolution of these items.

## Signatures

| Technician | Client |
| --- | --- |
| Signature: ___________________________________ | Signature: ___________________________________ |
| Printed Name: Sai | Printed Name: Tom |
| Date / Time: May 1, 2026 ¦ Arr: 8:15 AM ¦ Dep: 5:45 PM | Date / Time: _______________________________ |

Technijian, Inc.  •  Irvine, California  •  (949) 379-8499  •  CONFIDENTIAL — Technician Worksheet  •  Version: Updated with Full Visit Findings